NAT (Network Address Translation)

What it actually is:

NAT is a function on a router/firewall that translates private IP addresses to a public IP address.

Example:

Your laptop → 192.168.1.10

Your phone → 192.168.1.20

Router public IP → 103.x.x.x

When your laptop accesses google.com:

192.168.1.10 → router rewrites it → 103.x.x.x → Internet

The router keeps a translation table (NAT table) so replies go back to the correct internal device.

This is basic stateful packet tracking.

Why NAT exists:

IPv4 address exhaustion

Hide internal addressing scheme

Basic isolation of internal network

But understand this clearly:

NAT is NOT a firewall.

It is not a security control by design.

It just translates addresses.

Security benefit is incidental because unsolicited inbound traffic doesn’t know which internal host to go to (unless port forwarding is configured).

Real-world reality:

In enterprise networks, NAT is usually done on edge firewalls.

In cloud (AWS, Azure, GCP), NAT gateways allow private subnets to reach the internet without being directly reachable.

Types of NAT:

Static NAT – 1:1 mapping

Dynamic NAT – pool-based

PAT (Port Address Translation) – many-to-one (most common, home routers)

PAT is what most people call “NAT”.

VPN (Virtual Private Network)

What it actually is:

A VPN creates an encrypted tunnel between two endpoints over an untrusted network (usually the internet).

Key properties:

Encryption

Authentication

Integrity

Encapsulation

Protocols you should know:

IPsec

OpenVPN

WireGuard

SSL/TLS-based VPN

What a VPN does in practice:

Your traffic:

Device → encrypted tunnel → VPN server → Internet

To the outside world, your public IP becomes the VPN server’s IP.

Important correction:

A VPN hides traffic from:

Local network

ISP (to an extent)

But it does NOT hide you from:

The VPN provider

Websites (they still see cookies, fingerprinting, login identity)

Law enforcement with proper authority

If you log into Gmail through a VPN, Google still knows it’s you.

NAT vs VPN (Core Difference)

NAT:

Layer 3 address translation

No encryption

No identity abstraction

Mostly routing function

VPN:

Encrypted tunnel

Provides confidentiality + integrity

Extends private network over public infrastructure

They solve different problems.

Real-World Scenarios

Home network:

Router does NAT

You connect to VPN client → encrypted tunnel through NAT

Corporate network:

Employee laptop → VPN client → corporate firewall

Firewall performs NAT to internet

Cloud architecture:

Private EC2 → NAT Gateway → Internet

Remote admin → VPN → VPC

Security Misconceptions (Common Beginner Mistakes)

“NAT protects me.”

Not really. It’s not a security control framework like a firewall with ACL rules.

“VPN makes me anonymous.”

False. It changes your network location, not your identity.

“VPN prevents hacking.”

No. If your endpoint is compromised, VPN is irrelevant.

“Using VPN means I can’t be tracked.”

Browser fingerprinting, account login, behavioral profiling still apply.

Attack Perspective (MITRE ATT&CK Context)

NAT:

Irrelevant to most attacker techniques.

Attackers bypass NAT via reverse shells, C2 callbacks, phishing, etc.

VPN:

Common initial access vector (compromised VPN credentials).

Credential stuffing against VPN portals.

Exploiting VPN appliances (e.g., Fortinet, Pulse Secure vulnerabilities).

Used by attackers for lateral movement once inside.

Many real breaches started from exposed VPN services with weak MFA.

Defensive Considerations

If you deploy VPN:

Enforce MFA

Monitor login anomalies

Restrict split tunneling where possible

Log and analyze VPN auth events

Patch VPN appliances aggressively

If you rely on NAT:

Use proper firewall rules

Disable unnecessary port forwarding

Implement IDS/IPS

Do not assume “no public IP = safe”

Bottom Line

NAT = address translation

VPN = encrypted tunnel

They operate at different layers and solve different problems.