First, understand what you’re actually doing.

When you start a capture in Wireshark, you’re putting your network interface card (NIC) into capture mode using libpcap/WinPcap/Npcap. You’re not “hacking.” You’re passively sniffing packets that your interface can see.

Now the practical steps.

Step 1 — Run with proper privileges
 On Linux:

- Either run: sudo wireshark

- Or add your user to the wireshark group and relogin (preferred production approach)

On Windows:

- Install with Npcap.

- Run Wireshark as Administrator if required.

If you don’t have permissions, you won’t see interfaces or packets.

Step 2 — Choose the correct interface
 When Wireshark opens, it lists interfaces:

- eth0 / enp0s3 → wired

- wlan0 → Wi-Fi

- lo → loopback (127.0.0.1 traffic only)

- Docker / VM adapters if present

Pick the interface that actually carries traffic.
 If you're on Wi-Fi and you select Ethernet, you’ll capture nothing.

Step 3 — Start capture
 Double-click the interface.
 Traffic starts immediately.

You will see:

- Source IP

- Destination IP

- Protocol (TCP, UDP, DNS, HTTP, TLS, etc.)

- Length

- Info field

That’s raw packet-level visibility.

Step 4 — Generate traffic (so you see something)
 Open a browser and visit a website.
 Run:

- ping 8.8.8.8

- curl example.com

You should immediately see:

- DNS queries

- TCP 3-way handshake (SYN, SYN-ACK, ACK)

- TLS handshake if HTTPS

- ICMP if ping

If you don’t see anything:

- Wrong interface

- Firewall blocking capture

- VM network misconfiguration (very common in labs)

Step 5 — Use display filters (critical skill)

Without filters, you’ll drown in noise.

Examples:

Show only DNS:

dns

Show only HTTP:

http

Show only traffic to Google DNS:

ip.addr == 8.8.8.8

Show only TCP SYN packets:

tcp.flags.syn == 1 && tcp.flags.ack == 0

This is how analysts isolate signals during incident response.

Real-world note:

In enterprise networks:

- You won’t see other users’ traffic unless you're on a SPAN/mirror port.

- On switched networks, you only see your own broadcast + direct traffic.

- To capture across a subnet, you need port mirroring or network TAP hardware.

SOC reality:
 Most detection is done via:

- Network sensors (Zeek, Suricata)

- IDS/IPS

- NetFlow

- PCAP from SPAN ports

Wireshark is typically used for:

- Deep packet inspection

- Malware C2 analysis

- TLS handshake debugging

- Protocol troubleshooting

- Incident response packet validation

Common beginner mistakes:

- Capturing on loopback accidentally

- Forgetting HTTPS is encrypted (you won’t see payload)

- Not using capture filters vs display filters properly

- Capturing for too long and filling disk

Capture filter example (applied before starting):

port 80