sudo apt update

sudo apt install wireshark

During installation, you’ll be asked whether non-root users should be allowed to capture packets. If you choose “Yes”, your user gets added to the wireshark group.

Then:

sudo usermod -aG wireshark $USER

Log out and back in.

If you run it as root unnecessarily, you increase attack surface. Best practice: use dumpcap with proper permissions instead of full GUI root execution.

On Ubuntu/Debian:

Same process via apt.

On Windows:

Download the installer from the official Wireshark site. It installs:

• Wireshark GUI

• TShark (CLI version)

• Npcap (packet capture driver)

Npcap is critical. Without it, packet capture won’t work. Always install in WinPcap-compatible mode unless you know why you shouldn’t.

On macOS:

Install via Homebrew:

brew install wireshark

You must also install and approve the packet capture driver in system settings due to macOS security restrictions.

Operational advice:

In enterprise environments, you rarely run Wireshark directly on production servers. Instead, you:

• Capture via SPAN port on a switch

• Use tcpdump on a Linux server and export PCAP

• Analyze offline in Wireshark

For large captures, use TShark instead of the GUI. GUI will choke on multi-GB PCAPs.

Common beginner mistakes:

• Capturing on the wrong interface

• Forgetting to disable checksum validation errors (NIC offloading causes false positives)

• Assuming “no packets” means “no attack”

• Trying to use Wireshark as an IDS